SEDEQ: A SUITE OF PROGRAMS FOR ANALYSIS OF SEDIMENTATION

EQUILIBRIUM IN SOLUTIONS OF A SINGLE MACROSOLUTE IN THE CONTEXT

OF MODELS FOR EQUILIBRIUM SELF-ASSOCIATION AND/OR NONEQUILIBRIUM

MIXTURES OF INDEPENDENTLY SEDIMENTING SOLUTES

Allen P. Minton Laboratory of Biochemical Pharmacology National

Institute of Diabetes, Digestive, and Kidney Diseases National

Institutes of Health Bethesda, MD 20892

email: minton@helix.nih.gov

Version 4.0 - June 1995

The SEDEQ package contains the following files:

READ.ME - this file.

SEDEQ1B.EXE - a program for global non-linear least-squares

fitting of up to 3 equilibrium scans from the Beckman XLA

Analytical Ultracentrifuge to equilibrium self-association and/or

mixture models.

CALCSIG.EXE - a program for analyzing and parameterizing the

noise characteristics of the XLA.  This program is (1) useful in

comparing the performance of different centrifuges and with

factory spec, and (2) necessary to reliably perform weighted fits

with SEDEQ1B and determine the uncertainty of derived parameters.

SIMEQF.EXE - a program for simulating the results of

equilibrium experiments carried out on a self-associating

macrosolute, some fraction of which may be specified to be

incompetent to self-associate.  Normally-distributed

pseudo-random error may be added to the calculated results.

SIMMIX.EXE - a program for simulating the results of

equilibrium experiments carried out on a mixture of up to three

independently sedimenting (i.e., noninteracting) macrosolutes in

arbitrary proportion.  Normally-distributed pseudo-random error

may be added to the calculated results.

OFFBASE.EXE - a program for adding or subtracting a constant

numeric value from all OD readings in an XLA data file.

Ordinarily used for subtracting a constant baseline prior to data

analysis.

TEST???.RA1 - a set of synthetic data files to be used in

testing the fitting program SEDEQ1B.

CHYM1?.RA1 - a set of real data files on chymotrypsinogen,

obtained from the Beckman applications laboratory, to be used in

testing the fitting program SEDEQ1B.

GENERAL APPLICABILITY AND LIMITATIONS

1.  It is assumed that the solutions are ideal (i.e.,

macrosolutes are sufficiently dilute that repulsive

intermolecular interactions may be neglected.

2.  It is assumed that the solutions obey Beers' Law; i.e.,

the observed absorbance at a particular wavelength is linear in

the concentrations of each of the chemical species present in the

solution.

3.  Equilibrium association constants obtained via

least-squares fitting are expressed in units of inverse

absorbance units.  They may be readily converted to inverse molar

or weight concentrations via simple linear relations if the molar

or specific extinction coefficients are known.  Thus all scans

analyzed in a single global fit must be acquired at the same

wavelength, and differ only in total solute concentration and/or

rotor speed.

TREATMENT OF EXPERIMENTAL ERROR

It is assumed throughout that experimental uncertainty in

OD(r) may be characterized by the empirical relation

�(OD) = a0 + a1*OD

The values of a0 and a1 may be obtained for a given

ultracentrifuge using SIGCALC, which reads an XLA data file,

plots column 3 (the XLA estimate of �) as a function of column 2

(OD), and then obtains the least-squares straight line through

this plot.  Typical values obtained on two machines here at the

NIH are a0 = 0.003 and a1 = 0.005 at 280 nm and relatively low

rotor speeds (<15K rpm).  These numbers may vary with wavelength

and may possibly increase with increasing rotor speed.  You

should use SIGCALC on your own data sets to get your own

estimates of a0 and a1.

SIMULATION PROGRAMS

SIMEQF and SIMMIX create simulated XLA data files, including

pseudo-normally distributed noise with � defined by the

parameters a0 and a1 as described above.  SIMEQF is for solutions

containing monomer and up to 3 oligomers of a single

self-associating component.  Allowance may be made for some

fraction of the solute that is incompetent to self-associate and

sediments independently of any oligomers that may exist.  SIMMIX

is for solutions containing up to 3 independently sedimenting

solutes.  These programs are for generating data files that may

be used to test data analysis software.  For example, given a

certain level of noise, how well can different models or

parameter values be resolved?

BASELINE SUBTRACTION

Indeterminate baselines are highly correlated with model

parameters and are the source of much uncertainty in the value of

parameter values obtained via least squares fitting.  Allowing

baselines to vary independently may also permit false or

unsuitable models to spuriously "fit" a set of data.  I strongly

recommend that if possible, the baseline for each scan be

experimentally quantified (for example, by meniscus depletion)

and subtracted from each data set prior to analysis using the

program OFFBASE (OFFset BASEline). Then the value of the baseline

parameter in the model may be fixed equal to zero in the data

analysis, thus removing a potentially troublesome complication.

OFFBASE is a command-line driven program.  To see the required

syntax, type 'offbase' from the DOS prompt.

LEAST-SQUARES FIT OF DATA

SEDEQ1B can perform a least squares fit of up to three

equilibrium XLA scans to either of two models:

1. An equilibrium self-association model in which monomer can

exist in equilibrium with up to three oligomers.  Allowance can

be made for the existence of some fraction of solute that cannot

self-associate.  This fraction sediments as independent monomer

and does not equilibrate with oligomers.

2. A mixture model in which up to three non-interacting

macrosolutes in arbitrary proportion (of absorbance)

independently achieve sedimentation equilibrium.

SEDEQ1B is limited to the case in which all data sets

simultaneously fit to a single set of model parameters have the

same baseline.  Another version of this program, SEDEQ3B, is

under development that will permit different baselines for each

individual data set.

The fitting parameters in the model for equilibrium

association are M1, the molecular weight of monomer, log K2, log

K3, and log K4, the equilibrium association constants for

formation of dimer, trimer, and tetramer, base, the constant

baseline, and f comp, the fraction of total solute competent to

self associate.  A particular i-mer may be excluded from a model

being tested by constraining log Ki to be a large negative

number, say -20.  The baseline may be floated, but this is not

recommended, as discussed above.  Initially f comp should be

constrained equal to 1 to see whether a model is satisfactory.

Then the condition may relaxed to see whether the fit may be

improved thereby.

The fitting parameters in the nonequilibrium mixture model are

M1, M2, and M3, the molecular weights of the three solute

components, f2 and f3, the fractional absorbance contributions of

components 2 and 3 in the initial solution, and base, the

constant baseline. As in the self-association model, the

baseline(s) may be floated, but this is not recommended.

Experience indicates that much more reliable results may be

obtained for M2, f2, M3, and f3, if the value of M1 can be

obtained independently and constrained.

The initial value of a fitting parameter should not be set

identically to zero (if you want the best-fit value to be found

by fitting) because the algorithm used for fitting will not allow

it to change from identically equal to zero.  If you suspect that

the best-fit will be near zero, set the initial value equal to a

small number instead.

Reference concentrations are not fitting parameters in these

models. They are calculated from the data and values of the

independently variable parameters through the "conservation of

signal" algorithm described in Minton, A.P. "Conservation of

signal: a new algorithm for the elimination of the reference

concentration as an independently variable parameter in the

analysis of sedimentation equilibrium", in Schuster, T.M. & Laue,

T.M. (eds.), "Modern Analytical Ultracentrifugation", Birkhauser,

Boston, 1994.  This procedure not only lowers the number of

independently variable parameters, it greatly improves the

precision of parameter determination.

SEDEQ permits weighted fitting.  If this option is selected,

the user is prompted to enter the noise parameters ao and a1

specifying � as described above.  Then the program will seek to

minimize chi-sqr, defined as

chi-sqr = � ((y_i - ycalc_i)/�_i)^2

i

After finding the best fit, the program calculates the

probability that the best-fit value of chi-sqr could have been

attained by chance, given a normal distribution of error.  If

this probability exceeds a value determined by your confidence

limit then the fit is acceptable within that confidence limit.

For example if your confidence limit is 95% (meaning that you

don't want to risk more than a 5% chance of rejecting a valid

fit), then all fits yielding values of P(chi-sqr) exceeding 0.05

would be considered acceptable fits. Sometimes you will obtain

fits that appear to be quite good by visual inspection but give

unacceptably low values of P(chi-sqr). The usual reasons for this

are: 1. The values of ao and/or a1 that you have provided yield

unrealistically low estimates of � (i.e., unrealistically high

estimates of chi-sqr); and/or 2. The residuals are not normally

distributed, so these statistics are not appropriate.  The latter

could be symptomatic of either systematic error in the data or an

inadequate model.

SEDEQ1B permits limited editing of data sets.  You can remove

a block of "bad" points from either end or both ends of a set but

you cannot remove "bad" points in the middle of a set.

SEDEQ1B provides hard copy graphics output to Epson or IBM dot

matrix printers or HP Laserjet series printers, or to printers

that are graphics-compatible with any of the above.

If you are using a PC with an 80386 or earlier CPU, a math

coprocessor is strongly recommended.  The function minimizer will

work without a coprocessor, but VERY slowly.

TEST DATA SETS

TEST1??.RA1, TEST2??.RA1, and TEST3??.RA1 are three synthetic

data sets that represent three models.  Each set consists of

three scans at three different total loading concentrations

(absorbances). You will find that if you fit only a single data

file from a given set, the data are well-fit by either the

equilibrium self-association or non-equilibrium mixture model.

However, if you globally fit all three sets of data, you can

discriminate between the abilities of the respective models to

fit the different data sets.  Using SEDEQ1B you should be able to

determine which sets of data reflect equilibrium self-association

and which reflect a non-equilibrium  mixture model.  Also, in the

context of an equilibrium self-association model, you should be

able to ascertain whether the solute is fully competent to

self-associate, or whether a certain fraction of solute does not

self-associate.  After experimenting with unknown values of M1,

try repeating the fits constraining the value of M1 to 68000.

The results provide an example of the value of knowing the

monomer molecular weight independently.

CHYM1?.RA1 are a set of three genuine data files obtained on

three different concentrations of chymotrypsinogen in the Spinco

application laboratory.  Chymotrypsinogen is believed to exist as

an equilibrium mixture of monomer and dimer.  Can you confirm

this model with these data?  Can you exclude the presence of

non-negligible amounts of higher oligomrers?  Do the data reflect

the existence of a certain fraction of monomer that cannot

self-associate?

SUMMARY

The programs and ancillary files contained in this package can

be used to analyze sedimentation equilibrium data obtained from

solutions of a single macrosolute.  At the present time (June,

1995) we believe that this package is unique in several respects:

1. It is the only package that allows you to globally fit

multiple data sets with either equilibrium association or

nonequilibrium mixture models.

2. It is the only package that allows you to take into account

the possible existence of a fraction of "incompetent" solute that

cannot self-associate, but exists together with and is chemically

indistinguishable from native solute that can self-associate.

"Damaged" or partially denatured proteins having just these

properties are commonly encountered in real biochemical

preparations.

3. It is the only package that does not require the use of a

reference concentration as an independently adjustable parameter.

Feedback, constructive comments and suggestions for

improvement will be appreciated.

#################